BLE ATT Opcodes

Attribute Protocol (ATT) defines the request, response, command, notification, and indication messages used between the ATT client and ATT server over an established BLE connection.

| Opcode | Command Name | PDU | Description |
| --- | --- | --- | --- |
| 0x01 | ATT_ERROR_RSP | Error Response | Server response indicating that an ATT request could not be completed. It contains the opcode of the failed request, the relevant attribute handle, and an ATT error code. |
| 0x02 | ATT_EXCHANGE_MTU_REQ | Exchange MTU Request | Client requests the maximum ATT protocol data unit (PDU) size it can receive. The client sends its proposed ATT MTU. |
| 0x03 | ATT_EXCHANGE_MTU_RSP | Exchange MTU Response | Server returns its proposed ATT MTU. The effective ATT MTU is the smaller of the two exchanged values. |
| 0x04 | ATT_FIND_INFORMATION_REQ | Find Information Request | Client asks the server to return the attribute handles and UUID formats/UUIDs for attributes in a specified handle range. |
| 0x05 | ATT_FIND_INFORMATION_RSP | Find Information Response | Server returns the attribute handles and their UUIDs for the requested range. |
| 0x06 | ATT_FIND_BY_TYPE_VALUE_REQ | Find By Type Value Request | Client searches a handle range for attributes having a specified attribute type and attribute value. Commonly used during primary service discovery. |
| 0x07 | ATT_FIND_BY_TYPE_VALUE_RSP | Find By Type Value Response | Server returns matching handle ranges for the requested type and value. |
| 0x08 | ATT_READ_BY_TYPE_REQ | Read By Type Request | Client requests the values of all attributes of a specified type within a handle range. Commonly used to discover characteristics. |
| 0x09 | ATT_READ_BY_TYPE_RSP | Read By Type Response | Server returns the handles and values of the matching attributes. |
| 0x0A | ATT_READ_REQ | Read Request | Client requests the value of one attribute identified by its handle. |
| 0x0B | ATT_READ_RSP | Read Response | Server returns the value of the attribute requested by a Read Request. |
| 0x0C | ATT_READ_BLOB_REQ | Read Blob Request | Client requests part of a long attribute value, starting at a specified byte offset. |
| 0x0D | ATT_READ_BLOB_RSP | Read Blob Response | Server returns the requested portion of the attribute value. |
| 0x0E | ATT_READ_MULTIPLE_REQ | Read Multiple Request | Client requests the values of multiple attributes in one ATT transaction using a list of attribute handles. |
| 0x0F | ATT_READ_MULTIPLE_RSP | Read Multiple Response | Server returns the concatenated values of the requested attributes. |
| 0x10 | ATT_READ_BY_GROUP_TYPE_REQ | Read By Group Type Request | Client requests the attributes grouped by a specified group type. Commonly used for primary/secondary service discovery. |
| 0x11 | ATT_READ_BY_GROUP_TYPE_RSP | Read By Group Type Response | Server returns grouped attribute information, including handle ranges and group values. |
| 0x12 | ATT_WRITE_REQ | Write Request | Client requests that the server write a value to an attribute. This is acknowledged by a Write Response. |
| 0x13 | ATT_WRITE_RSP | Write Response | Server acknowledges a successful Write Request. |
| 0x16 | ATT_PREPARE_WRITE_REQ | Prepare Write Request | Client sends a portion of a value together with an offset. Used to queue fragments before a long write is committed. |
| 0x17 | ATT_PREPARE_WRITE_RSP | Prepare Write Response | Server acknowledges and echoes the prepared write information. The queued data is not committed yet. |
| 0x18 | ATT_EXECUTE_WRITE_REQ | Execute Write Request | Client asks the server either to commit all queued prepared writes or cancel them. |
| 0x19 | ATT_EXECUTE_WRITE_RSP | Execute Write Response | Server confirms successful execution/cancellation of the queued prepared writes. |
| 0x1B | ATT_HANDLE_VALUE_NTF | Handle Value Notification | Server sends an attribute value to the client without requiring a response. Commonly used for GATT notifications. |
| 0x1D | ATT_HANDLE_VALUE_IND | Handle Value Indication | Server sends an attribute value that requires a confirmation from the client. Commonly used for GATT indications. |
| 0x1E | ATT_HANDLE_VALUE_CFM | Handle Value Confirmation | Client confirms receipt of a Handle Value Indication. |
| 0x20 | ATT_READ_MULTIPLE_VARIABLE_REQ | Read Multiple Variable Length Request | Client requests multiple attribute values where the individual values may have different lengths. |
| 0x21 | ATT_READ_MULTIPLE_VARIABLE_RSP | Read Multiple Variable Length Response | Server returns the requested variable-length attribute values, each represented with its length. |
| 0x22 | ATT_MULTIPLE_HANDLE_VALUE_NTF | Multiple Handle Value Notification | Server sends multiple attribute handle/value pairs in a single notification PDU. |
| 0x52 | ATT_WRITE_CMD | Write Command | Client writes an attribute without requiring a response. This is the ATT mechanism behind GATT Write Without Response. |
| 0xD2 | ATT_SIGNED_WRITE_CMD | Signed Write Command | Client writes an attribute without a response while carrying an authentication signature. Historically useful for authenticated writes without an established encrypted connection. |

Missing values: 0x14, 0x15, 0x1A and 0x1C are reserved. (They exist but no ATT command is assigned to them in the specification).

Common GATT operations and their ATT opcodes

GATT Read: 0x0A Read Request → 0x0B Read Response

GATT Write: 0x12 Write Request → 0x13 Write Response

GATT Write Command: 0x52

GATT Notification: 0x1B Handle Value Notification

GATT Indication: 0x1D Handle Value Indication → 0x1E Handle Value Confirmation

Service Discovery: 0x10 Read By Group Type Request → 0x11 Response

Characteristic Discovery: 0x08 Read By Type Request → 0x09 Response